ECE 874 - Spring 2013             Test 3 Out-of-Class Component                Name ______________________

(20 points total)

Instructions:

- The test is due on Wednesday April 3 in class at 11:15AM.

- You must show all steps in your solutions

- Your exam solutions are to be your own work, you are not to give or receive assistance of any kind on this exam.

Given the system:

The desired trajectory is given as . Assume a=2 when simulating the system (you don’t use this value in the adaptive or robust controller) and assume you know it is a positive constant when designing the controllers.  Design and simulate the following tracking controllers. Plot tracking error and control input for each. Assume x(0)=0 in the simulation.

- (6 pts) Design and simulate an exact-model knowledge tracking controller.

- (6 pts) Design and simulate an adaptive tracking controller.

- (6 pts) Design and simulate a sliding-mode robust control. Assume that the parameter "a" is upper and lower bounded by 20%, ie assume you know 1.6<a<2.4. With that you should create a bound on the unknown f(x) = asin(x).

- (2 pts) In a paragraph, compare and contrast the performance of the three controllers.